#

File: teac.info,  Node: Top,  Prev: (sony.info),  Next: (technics.info),  Up: (remotes.info)

******************************************************************************

TEAC

******************************************************************************

Version:                3.2

Date:                   10 feb 94

* Menu:

* Technical info::

* CD::

#

File: teac.info,  Node: Technical info,  Prev: Top,  Up: Top

******************************************************************************

TECHNICAL INFO

******************************************************************************

Code length:            32 bits

Carrier:                39.2kHz

The code is only transmitted once, and after that a stop code is transmitted.

+--------------------------------+

Header:      |                                |

+                                +----------------+..

16T                       8T

8.8ms                     4.4ms

+--+

1 is coded:  |  |

..+  +------+..

T   3T

+--+

0 is coded:  |  |

..+  +--+..

T  T

+--+

|  |

After the last bit a ( ..+  +  ) is transmitted to terminate the last bit.

T

+--------------------------------+        +--+

Stop code:    |                                |        |  |

+                                +--------+  +

16T                  4T     T

8.8ms                2.2ms

T = 550us

Space between transmissions: 95000us between the headers of the codes.

To use the code (not with the remote package):

GROUP * 10000h + (Code * 100h) + (Code XOR FFh)

For more information please read the documentation.

#

File: teac.info,  Node: CD,  Prev:  Technical info,  Up: Top

******************************************************************************

CD

******************************************************************************

GROUP:

Code    Function

----    --------