April 24, 1997

========================================================================

Thanks to Lance Rose for sharing his results in decoding the format of

SA3 startup files.

========================================================================

DISCLAIMER:

As part of it's functionality, g7to creates MapDoc (*.sa3) files from

GPS data.  You can then display the data overlaid on a SA3 map, but it

is not real time and should not be used for navigation.

In my opinion, the Lat/Long grid shown in SA3 is not accurate enough for

critical navigtion applications (and the license agreement appears to

prohibit using maps from SA3 for navigation).  For instance, in the

Corvallis area of Oregon SA3 is in error about 150 feet to the East at

magnification 16.  For example, plotting a known Lat/Long on SA3 results

in a point on the SA3 map that is ~150 feet East of it's actual

location.  The error varies from place to place and with the magnitude

of the displayed map.  I've heard rumors that SA3 is up to 1/4 mile off

in some areas, but I've not personally observed an error that large.

There are no guarantees expressed or implied with this g7to and/or it's

documentation.  The package works for me, it may or may not work for

anyone else.

========================================================================

NOTE:  For this program, 'download' is defined as 'from the GPS to the

PC' and 'upload' is 'from the PC to the GPS'.  All references to

the GPS 45 apply to the 12XL.  This program may work with other

Garmin receivers, but it has not been tested by the author.

Versions greater than 2.04 have been tested on the 12XL only.

G7TO will upload/download Track/Waypoint/Route data to/from a Garmin

GPS45.  Data from the GPS45 may be displayed, redirected to a text

file, and/or sent to a Street Atlas 3.0 startup file, otherwise

known as MapDocs.

Almanac data from the GPS45 may be downloaded, but not uploaded.

The current position and time can be retrieved from the GPS45.

This function can be set to repeat every 'n' seconds.

Proximity waypoints are supported, but transfer is only between the

GPS and the text file format.  Proximity waypoints may not be sent

to Proj, Garmin64 or .SA3 files.  The distance parameter stored in

the text file is in meters.

Supported Input Formats:

1.  Text, GarDown7 raw format, Proj format, Garmin64 format,

navigate.exe routes file: routes.clg.

If the input file is Garmin64 or Proj format only the Track

and Waypoint records are processed.

2.  SA3 startup files (fname.sa3).  All symbols are output as

Waypoints.  The first 6 characters of an object's name

becomes the waypoint name.  The 7th character is skipped.

The 8th->Xth character becomes the waypoint comment.  The

waypoint time is taken from the time stamp on the input

file.

X==16 for Text output.

X==19 for PROJ output format.

X==20 for Garmin64 output format.

X==23 for Control Point Record (*80*) format.

3.  The GPS 45 via COM port.  Standard IRQ/IO supported.  The

GPS 45 I/O SETUP should be set to GRMN/GRMN and HOST.

Supported Output Formats:

1.  SA3 startup files (fname.sa3).  The default object is a

small circle.  The default text location is NE of the

object.  The '.sa3' suffix is not added by the program, it

must be input on the command line.

2.  Text file.

3.  PROJ.EXE and/or NAD2NAD.EXE format.  See the documentation

for PROJ for further information

4.  Control Point Record (*80*) for use with the program

UTMS.EXE.  See UTMS documentation for further information.

5.  Garmin64 format.  See the GARMIN64 documentation for an

explanation of this format.

6.  The GPS 45 via COM port, the file name MUST be G45x.  See

below for options on '-i G45x'.

7.  Navigate.exe routes.clg routes file. (Input cannot be SA3)

===========================================================================

Command line help information:

c:\g7to

GPS45 Upload/Download/File Conversion 2.06 Apr 24 1997 10:42:35 PST

USAGE:g7to -123 -d -v -h -i infile -p68t [-abceswnm -o outfile | >outtext]

For more information:

-h I for input help

-h O for output help

-h D for SA3 symbol attributes

-d sum Track distances if source is GPS45 or Text file

-v print stats for COM port

---------------------------------------------------------------------------

C:\g7to -h i

Allowable Input files:

SA3, text, [Proj, Garmin64 (Waypoint/Track only)], route.clg or GPS45

Use -1 or -2 to set standard COM Port 1-2

To specify the port parameters use:

-3 Port,IOaddr,IRQ (no spaces allowed) where:

Port is 1-4, IOaddr in hex (0xnnn) or decimal, IRQ (3-15) decimal

g7to -3 1,0x3f8,4       defines a standard COM1 port

Use:

-i G45Pn -- Send Time/Position record every n seconds (press 'x' to stop)

-i G45A  -- Send Almanac data

-i G45I  -- Send current time

-i G45D  -- Send software ID

-i G45R  -- Send Routes (output cannot be SA3)

-i G45T  -- Send Tracks (-d to sum distances)

-i G45W  -- Send Waypoints

-i G45X  -- Send Proximity Waypoints

-e Do not print symbol attributes to text file

(SA3 input, text out only)

---------------------------------------------------------------------------

C:\g7to -h o

Output can be to SA3 file or the GPS45 (-o G45) and stdout

-o G45O turns off the GPS45

-----------------------------------------------------------------

-o <fname> - name for Street Atlas 3.0 file

-a,-b,-c,-g,-l,-m,-n,-s,-w used with .sa3 output only

a - Text alignment:  NW N NE W E SW S SE

b - Object Code.  Values 0-52 see G7TO.DOC

c - ?color ?=T for Text color, O for Object Color

Allowable colors: black,red,blue,green,yellow

g - map magnitude

l - 1st-center map Lat|2nd-center map Long (DDdMM.mmmm)

m - Merge -i (SA3 file) and -m (SA3 file) to -o (SA3 file)

n - No name and no description

s - ?size ?=T for Text size, O for Object size

Allowable sizes: vs,s,m,l,vl

w - Waypoint name only, no description

-r - Navigate.exe Route file, routes.clg

-----------------------------------------------------------------

or stdout (can re-direct to a text file)

T - Text Format

P - Proj.4.3.3 Record    (proj.exe, nad2nad.exe)

6 - Garmin64 Format      (garmin.exe)

8 - Control Point Record (utms.exe)

---------------------------------------------------------------------------

C:\g7to -h d

-- OBJECTS --

Size     : vs, s, m, l, vl

Alignment: nw, n, ne, w, e, sw, s, se

Color    : black, red, blue, green and yellow

Codes for use with -b:

0 = Agricultural                            35 = One Way Up

1 = Airport        18 = Education           36 = Park

2 = Amusement      19 = Emergency           37 = Parking

3 = Arrow, Down    20 = Fishing             38 = Phone

4 = Arrow, Left    21 = Fuel                39 = Picnic

5 = Arrow, Right   22 = Golf Course         40 = Post Office

6 = Arrow, Up      23 = Government Building 41 = Power

7 = Bank           24 = Historic Site       42 = Religious Star

8 = Beach          25 = Home                43 = Religious Cross

9 = Boat Ramp      26 = Industrial          44 = Restaurant

10 = Campground     27 = Information         45 = Sports

11 = Cemetary       28 = Library             46 = Square

12 = Circle         29 = Lodging             47 = Star

13 = Construction   30 = Mountain            48 = State Capitol

14 = Cross          31 = National capitol    49 = Stop Sign

15 = Delivery Stop  32 = One Way Down        50 = Triangle Down

16 = Diamond        33 = One Way Left        51 = Triangle Up

17 = Doctor/Dentist 34 = One Way Right       52 = Zoo

===========================================================================

Typical usage:

INPUT -- GPS45 output .sa3 and text.  (a download)

When file input or output is from the GPS45 and the output or input is

to a SA3 file a text output can also be generated.

g7to -i G45W -o waypoint.sa3 -t >waypoint.txt

will download waypoint data from the GPS45 and store it in the SA3

startup file waypoint.sa3 and also write it to the text file

waypoint.txt.

g7to -o G45 -i waypoint.sa3 -t >waypoint.txt

uploads waypoint data to the GPS45 from the SA3 startup file

waypoint.sa3 and also writes it to the text file waypoint.txt.

If no output file is specified then the output is assumed to be text.

If a COM port is required and none is specified then COM1 is assumed.

------------------------------------------------------------------------

INPUT -- text format, OUTPUT SA3 Text is blue, Text size medium

g7to -i waypoint.txt -o waypoint.sa3 -c Tblue -s Tm

------------------------------------------------------------------------

INPUT -- .sa3, OUTPUT -- CPR (*80*) format

g7to -i myfile.sa3 -8

will read the SA3 startup file myfile.sa3 and print its contents to

the display in Control Point Record format.

g7to -i myfile.sa3 -8  >myfile.80

will redirect the output to myfile.80

------------------------------------------------------------------------

INPUT -- SA3 file, output textfile, output an attribute command

line each time an attribute changes.

g7to -i waypoint.sa3 -o waypoint.txt

The 'command line' output will be similar to:

! -b 12 -c oblack -c tblue -s ovs -s tm -a one

This line signifies that all points in the file following the

command line will be object #12 (circle) the circle will be

black, the text will be blue, the object will be very small and

the text size will be medium.  The text alignment will be North

East of the object.

g7to -i waypoint.sa3 -o waypoint.txt -e

This command line will process the .SA3 file but all objects

will be the default object.

------------------------------------------------------------------------

The 'sa3 output only' options -n,-w,-a,-b,-s,-m,-g,-l and -c:

-w - The text records in the output sa3 file will be blank.

This makes it easier to see the map surrounding a point,

but the user must rely on the 6 character waypoint name for

waypoint identification.  This works with GarDown7 or text

input, not on a merge.

-n - Points from a track file do not contain text information,

and the 'waypoint name' normally becomes the position in

the source file.  For instance, if you have 10 points in

your track source file then the points are 'named' 1-10.

-n suppresses this number and only the default symbol, a

small circle, is displayed.  -n also overrides the -w

option and on a waypoint will pass on only the default

symbol.  This works with GarDown7 or text input, not on a

merge.

-a <align> - Text alignment valid parameters are: NW N NE W E SW S SE

-c [T|O]<color> - Object and Text color.  First character is a

'T' for Text color or 'O' for object color.  Valid

colors are black,red,blue,green,yellow.

-s [T|O]<size> - Object and Text size.  First character is a 'T'

for Text size or 'O' for object size.  Valid sizes are

vs,s,m,l,vl (for very small, small, medium, large, very

large).

-b <object#> - See below for valid object codes.

-m <file> - SA3 file '-i <file>' and SA3 file '-m <file>'

are merged into SA3 file '-o <file>'

-g <maGnitude> Magnitude of the output .sa3 file

-l <Lat> -l <Long> - The first time the -l parameter appears on

the command line it is assumed to be the desired latitude of

the center of the .sa3 output file.  The second time it

appears in the command line it is taken to be the Longitude

of the center of the .sa3 file.  The input format is

DDDdMM.mmmm

NOTE:

If the input file is text/gardown7 format the program will calculate

the approximate values for the center and magnification of the SA3

map that will cover the points.  This calculation is performed at the

time the '-i fname' is scanned on the input line.  Therefore, if -g

and/or -l -l entries follow the '-i fname' parameter they will

override the calculated values.  For example:

g7to -i fname -o xxx.sa3 -g 12 -l N44d44.0 -l W122d45.0

will use use the parameters on the input line while

g7to -o xxx.sa3 -g 12 -l N44d44.0 -l W122d45.0 -i fname

will use the calculated values.  This calculation is designed to

return a magnification that will be slightly smaller than might be

shown on a single page, therefore, all points should be displayed.

===========================================================================

Waypoint storage/retrieval accuracy.

The GPS45 sends/receives latitude/longitude data encoded in 4 byte

'long' variable.  However, the last byte of this variable is always

rounded by the GPS45.  Because of this there will be times when the

waypoint downloaded from the GPS45 will differ from that sent to it.

For instance if the waypoint:

W  TESTP2 N44 35.8931 ???????????? Mon Mar 25 15:28:04 1996 TEST WAYPOINT

is sent to the GPS45 it will be returned as

W  TESTP2 N44 35.8929 ???????????? Mon Mar 25 15:28:04 1996 TEST WAYPOINT

This 'rounding' applies to both the latitude and longitude variables.

===========================================================================

.TXT file format

The .txt file created by this program can contain Waypoint data,

proximity Waypoint data, Track data Route start, Route End and datum

records.  This program assumes WGS84 datum, and therefore the datum

record is informational only, it does not trigger a conversion.  The

format for the various record types is shown below.

Character positions are shown for clarity.

Route begin/end and Map datum records are stripped when the output is a

.sa3 file.

The datum for SA3 is WGS 84, and this program converts Lat/Lon data

literally, so the GPS must be using WGS 84/NAD83 or accuracy will be

degraded.

.TXT output format

Map datum:

111111111

123456789012345678

D  100	WGS 84

Waypoint, Proximity Waypoint and Track:

11111111112222222222333333333344444444445555555555666666666677777777778

12345678901234567890123456789012345678901234567890123456789012345678901234567890

N  New Track Start

W  YAQBRG N44 37.3259 W124 03.4229 Sun Mar 17 04:02:26 1996 YAQUINA BAY BRDG

P  YAQBRG N44 37.3259 W124 03.4229 Sun Mar 17 04:02:26 1996 YAQUINA BAY BRDG 234.5

T  N44 35.9589 W123 15.4792 Sun Jan 21 22:47:51 1996

! -b 12 -c oblack -c tblue -s ovs -s tm -a one

Route begin:

11111111112222222222333333333344444444445555555555666666666677777777778

12345678901234567890123456789012345678901234567890123456789012345678901234567890

R  01 ROUTE NAME

Route end:

123456

E  01

===========================================================================

Proj format allowed on input (waypoint is output):

0123456789012345678921234567893123456789412345678951234567896123456789712345

124d04'36.28800"W 44d40'31.30200"N 1996-03-25 15:28 YAQHED YAQUINA HEAD

^           ^ ^

|           | |

must be 'd'    | |

must be '"'----. |

|

TAB or SPACE-----.

Proj format allowed on input (Track point is output):

0123456789012345678921234567893123

124d04'36.28800"W 44d40'31.30200"N

^           ^ ^

|           | |

must be 'd'    | |

must be '"'----. |

|

TAB or SPACE-----.

===========================================================================

Time:  If input is from a sa3 file, time is taken from the time stamp of

the input (-i) file.  For gardown7 or text input the time is

taken from the input record.  The GPS45 ignores the time stamp on

upload of track information.

===========================================================================

Distance:  The track distance calculation is *not* rigorous, it:

1.  Assumes that one minute of latitude is equal to 1 nautical mile.

2.  Assumes that longitude distance is related to latitude distance

by the Cosine function.

3.  Calculates the distance between two points by taking the

square root of the sum of the squares of the difference in

latitude and longitude.  Nautical miles are then converted to

statute mile by multiplying by 1.15078 and to kilometers by

multiplying by 1.852.

4.  Effects of SA are not removed.  A stationary track under SA

influence will cover quite a bit of ground.

5.  Distance is also calculated for Waypoints (but not Proximity

Waypoints), however as waypoints are usually stored in

alphabetical rather than chronological order the distance

calc is likely to be meaningless.

===========================================================================

.sa3 notes:  Street Atlas 3 has at least 3 objects that can be stored in

the .sa3 file.  These objects are Text, Place and MapNotes.

Places are very similar to waypoints, i.e.  there is a Lat/Long

location and a name.  All waypoints in a .txt file or from the

GPS45 become Places during a conversion to .sa3.

All three .sa3 objects are converted to waypoints during a .sa3

to GPS45 upload and/or a .txt conversion (and therefore would

become Places if the .txt file were used to create a new .sa3

file).

During a merge of two .sa3 files the objects are unchanged.

===========================================================================

Version History:

-------------------------------------------------------------------------------

2.06    - Added Proximity waypoint upload/download.  This is for Garmin

to text and text to Garmin only.

-------------------------------------------------------------------------------

2.05    - Navigate.exe waypoint names are now converted to

Garmin format by first removing all ' ' and '#'

characters.  Earlier versions of g7to replaced these

characters with a '-'.  If the resulting name is

longer than 6 characters, the program takes the first

3 and the last 3 characters of the name.  This allows

default waypoints in the shareware version of

Navigate.exe to be unique.

-------------------------------------------------------------------------------

2.04    - Fixed a Garmin 64 formatting bug.

-------------------------------------------------------------------------------

2.03    - The rs232 port on a 100/200LX was not turned off at the end

of data collection.  It is now.

- Fixed waypoint name length problem when input is proj

format

-------------------------------------------------------------------------------

2.02    - Added ability to convert routes between Paul Mouland's

navigate.exe (routes.clg) and text and/or the G45.

- Misc tidying up.  Informs GPS 45 of number of records that

will be sent during an upload.

-------------------------------------------------------------------------------

2.01    - Fixed problem with Garmin Protocol...end of frame was not

decoded properly in all cases.

- Routes had an 'E  XX' record in all output formats, not just

text.  Output of 'E  XX' record only in TEXT mode now

- Fixed bug in reading MapNotes on .SA3->txt conversion

- now uploads Almanac data.  The Almanac file format is now

the same as that found at:

http://satnav.atc.ll.mit.edu/gps/current.gpy.   On download

from the GPS45 a week# will be -1 if the satellite isn't

healthy.  This function assumes that the satellites are in

order in the file.  Missing sats are ok, but the ID number

must be correct.

-------------------------------------------------------------------------------

2.0     - Now uploads/downloads Waypoints, Tracks, Routes and other

items directly from/to the GPS45.  It has not been tested

on other Garmin Units.

- Sums track distances using a simple formula.

- Expanded object attribute capability on conversion to/from

SA3 files.

- Works properly on HP 100LX/HP 200LX palmtop units,

i.e.  turns on COM1 as wire, disables light sleep,

transfers data, turns off COM1, enables light sleep.

- Supports standard COM1/COM2 ports or you can define your

own IRQ and IO address.

-------------------------------------------------------------------------------

1.41	- fixed round off error

-------------------------------------------------------------------------------

1.4 	- added optional magnitude, latitude and longitude parameters

to be used for the center of the output .sa3 file

- changed os to -o in two places in DOC file.

- fixed broken .sa3 -> g64 format.  The time was expressed in

4 digits rather than the expected 2 digits (1996 vs 96)

-------------------------------------------------------------------------------

1.3 	- fixed time related bugs

- fixed MapNote,Symbol,Text problem in merge/output

- cleaned up source code and doc file

-------------------------------------------------------------------------------

1.2 	- added -m option to merge 2 sa3 format files into a

third sa3 file

- changed reference to garmin7 to gardown7

-------------------------------------------------------------------------------

1.1 	- Changed compile model from Large to Small

- Added ability to set object type, size and color when

output is a .SA3 file

- Added ability to set text size, color and alignment when

output is a .SA3 file

- Fixed code which allowed a waypoint with an interior

' ' (space), i.e.  'MY PNT'.  The space is changed to a '-',

i.e. 'MY-PNT'.

- More descriptive error output for improper option input

-------------------------------------------------------------------------------

1.0	As released

-------------------------------------------------------------------------------

A sample Almanac data file is shown between the 'Cut HERE' lines.

This sample file was created by

1.  Retrieve the data for week 851 from

http://satnav.atc.ll.mit.edu/gps/current.gpy and save it as file

current.gpy.

2.  Upload current.gpy to the GPS45 by: g45to -i current.gpy -o g45

3.  Download the sample file by g7to -i g45a >sample.fil

--------------------Cut HERE----------------------------

**** Week 902 almanac for PRN-01 ***********

ID:                          001

Health:                      000

Eccentricity:                3.796577454e-03

Time of Applicability(s):    4.055040000e+05

Orbital Inclination(rad):    9.544380903e-01

Rate of Right Ascen(r/s):   -8.114623640e-09

SQRT(A) (m^1/2):             5.153667480e+03

Right Ascen at TOA(rad):    -8.868364096e-01

Argument of Perigee(rad):   -1.571425915e+00

Mean Anom(rad):              1.527377486e+00

Af0(s):                      1.811981201e-05

Af1(s/s):                    0.000000000e+00

week:                        902

**** Week 902 almanac for PRN-02 ***********

ID:                          002

Health:                      000

Eccentricity:                1.643037796e-02

Time of Applicability(s):    4.055040000e+05

Orbital Inclination(rad):    9.424658418e-01

Rate of Right Ascen(r/s):   -8.194627199e-09

SQRT(A) (m^1/2):             5.153630859e+03

Right Ascen at TOA(rad):     1.142113447e+00

Argument of Perigee(rad):   -2.374259233e+00

Mean Anom(rad):             -2.745677948e+00

Af0(s):                     -3.566741943e-04

Af1(s/s):                   -3.637978807e-12

week:                        902

**** Week 902 almanac for PRN-03 ***********

ID:                          003

Health:                      000

Eccentricity:                2.902507782e-03

Time of Applicability(s):    4.055040000e+05

Orbital Inclination(rad):    9.528442025e-01

Rate of Right Ascen(r/s):   -8.377492478e-09

SQRT(A) (m^1/2):             5.153602051e+03

Right Ascen at TOA(rad):     2.220814228e+00

Argument of Perigee(rad):    2.501389265e+00

Mean Anom(rad):             -1.835575104e-01

Af0(s):                      7.629394531e-06

Af1(s/s):                    3.637978807e-12

week:                        902

**** Week 902 almanac for PRN-04 ***********

ID:                          004

Health:                      000

Eccentricity:                4.288196564e-03

Time of Applicability(s):    4.055040000e+05

Orbital Inclination(rad):    9.746614695e-01

Rate of Right Ascen(r/s):   -7.600316820e-09

SQRT(A) (m^1/2):             5.153734375e+03

Right Ascen at TOA(rad):    -2.990158081e+00

Argument of Perigee(rad):   -9.046850204e-01

Mean Anom(rad):             -4.207084477e-01

Af0(s):                      2.479553223e-05

Af1(s/s):                    3.637978807e-12

week:                        902

**** Week 902 almanac for PRN-05 ***********

ID:                          005

Health:                      000

Eccentricity:                9.765625000e-04

Time of Applicability(s):    4.055040000e+05

Orbital Inclination(rad):    9.452581406e-01

Rate of Right Ascen(r/s):   -8.171769039e-09

SQRT(A) (m^1/2):             5.153620117e+03

Right Ascen at TOA(rad):     1.158461452e+00

Argument of Perigee(rad):   -1.084146142e+00

Mean Anom(rad):              4.155567288e-01

Af0(s):                      9.346008301e-05

Af1(s/s):                    0.000000000e+00

week:                        902

**** Week 902 almanac for PRN-06 ***********

ID:                          006

Health:                      000

Eccentricity:                6.424903870e-03

Time of Applicability(s):    4.055040000e+05

Orbital Inclination(rad):    9.598848820e-01

Rate of Right Ascen(r/s):   -8.286059838e-09

SQRT(A) (m^1/2):             5.153712891e+03

Right Ascen at TOA(rad):     2.254529238e+00

Argument of Perigee(rad):   -2.674580097e+00

Mean Anom(rad):              4.202058613e-01

Af0(s):                      4.768371582e-06

Af1(s/s):                    0.000000000e+00

week:                        902

**** Week 902 almanac for PRN-07 ***********

ID:                          007

Health:                      000

Eccentricity:                8.700370789e-03

Time of Applicability(s):    4.055040000e+05

Orbital Inclination(rad):    9.622877240e-01

Rate of Right Ascen(r/s):   -8.274630758e-09

SQRT(A) (m^1/2):             5.153676758e+03

Right Ascen at TOA(rad):     2.220971346e+00

Argument of Perigee(rad):   -2.335334778e+00

Mean Anom(rad):              2.333193302e+00

Af0(s):                      7.438659668e-04

Af1(s/s):                    0.000000000e+00

week:                        902

**** Week 902 almanac for PRN-09 ***********

ID:                          009

Health:                      000

Eccentricity:                5.961418152e-03

Time of Applicability(s):    4.055040000e+05

Orbital Inclination(rad):    9.450424314e-01

Rate of Right Ascen(r/s):   -7.840326610e-09

SQRT(A) (m^1/2):             5.153609375e+03

Right Ascen at TOA(rad):     1.315190047e-01

Argument of Perigee(rad):    1.675922722e-01

Mean Anom(rad):              2.965324223e-01

Af0(s):                     -3.528594971e-05

Af1(s/s):                    0.000000000e+00

week:                        902

**** Week 902 almanac for PRN-10 ***********

ID:                          010

Health:                      000

Eccentricity:                2.188682556e-03

Time of Applicability(s):    4.055040000e+05

Orbital Inclination(rad):    9.624674916e-01

Rate of Right Ascen(r/s):   -7.863184770e-09

SQRT(A) (m^1/2):             5.153716309e+03

Right Ascen at TOA(rad):    -1.960087061e+00

Argument of Perigee(rad):   -3.739336729e-01

Mean Anom(rad):             -2.595160246e+00

Af0(s):                      6.675720215e-06

Af1(s/s):                    0.000000000e+00

week:                        902

**** Week 902 almanac for PRN-14 ***********

ID:                          014

Health:                      000

Eccentricity:                1.474857330e-03

Time of Applicability(s):    4.055040000e+05

Orbital Inclination(rad):    9.679203033e-01

Rate of Right Ascen(r/s):   -7.794610291e-09

SQRT(A) (m^1/2):             5.153563477e+03

Right Ascen at TOA(rad):    -1.917423129e+00

Argument of Perigee(rad):   -3.060511827e+00

Mean Anom(rad):              2.246802807e+00

Af0(s):                      1.811981201e-05

Af1(s/s):                    0.000000000e+00

week:                        902

**** Week 902 almanac for PRN-15 ***********

ID:                          015

Health:                      000

Eccentricity:                6.914615631e-03

Time of Applicability(s):    4.055040000e+05

Orbital Inclination(rad):    9.796229005e-01

Rate of Right Ascen(r/s):   -7.566029581e-09

SQRT(A) (m^1/2):             5.153558594e+03

Right Ascen at TOA(rad):    -2.952930212e+00

Argument of Perigee(rad):    1.607483029e+00

Mean Anom(rad):             -1.168938994e+00

Af0(s):                      4.177093506e-04

Af1(s/s):                    3.637978807e-12

week:                        902

**** Week 902 almanac for PRN-16 ***********

ID:                          016

Health:                      000

Eccentricity:                2.003669739e-03

Time of Applicability(s):    4.055040000e+05

Orbital Inclination(rad):    9.643310308e-01

Rate of Right Ascen(r/s):   -7.863184770e-09

SQRT(A) (m^1/2):             5.153657715e+03

Right Ascen at TOA(rad):    -1.910519838e+00

Argument of Perigee(rad):   -2.077315599e-01

Mean Anom(rad):             -1.123723745e+00

Af0(s):                      9.536743164e-06

Af1(s/s):                    0.000000000e+00

week:                        902

**** Week 902 almanac for PRN-17 ***********

ID:                          017

Health:                      000

Eccentricity:                9.140014648e-03

Time of Applicability(s):    4.055040000e+05

Orbital Inclination(rad):    9.807434678e-01

Rate of Right Ascen(r/s):   -7.520313261e-09

SQRT(A) (m^1/2):             5.153608887e+03

Right Ascen at TOA(rad):    -2.916248322e+00

Argument of Perigee(rad):    2.448255062e+00

Mean Anom(rad):              1.932538599e-01

Af0(s):                     -1.468658447e-04

Af1(s/s):                    0.000000000e+00

week:                        902

**** Week 902 almanac for PRN-18 ***********

ID:                          018

Health:                      000

Eccentricity:                6.491184235e-03

Time of Applicability(s):    4.055040000e+05

Orbital Inclination(rad):    9.432987571e-01

Rate of Right Ascen(r/s):   -8.228914439e-09

SQRT(A) (m^1/2):             5.153624023e+03

Right Ascen at TOA(rad):    -9.422645569e-01

Argument of Perigee(rad):    1.622361422e+00

Mean Anom(rad):              2.264727116e+00

Af0(s):                      2.574920654e-05

Af1(s/s):                    0.000000000e+00

week:                        902

**** Week 902 almanac for PRN-19 ***********

ID:                          019

Health:                      000

Eccentricity:                2.915859222e-03

Time of Applicability(s):    4.055040000e+05

Orbital Inclination(rad):    9.285162091e-01

Rate of Right Ascen(r/s):   -8.023191889e-09

SQRT(A) (m^1/2):             5.153700684e+03

Right Ascen at TOA(rad):     8.992774785e-02

Argument of Perigee(rad):   -2.989789486e+00

Mean Anom(rad):             -3.710447252e-01

Af0(s):                      2.307891846e-04

Af1(s/s):                   -7.275957614e-12

week:                        902

**** Week 902 almanac for PRN-21 ***********

ID:                          021

Health:                      000

Eccentricity:                1.328182220e-02

Time of Applicability(s):    4.055040000e+05

Orbital Inclination(rad):    9.617544413e-01

Rate of Right Ascen(r/s):   -7.863184770e-09

SQRT(A) (m^1/2):             5.153615723e+03

Right Ascen at TOA(rad):    -1.953871012e+00

Argument of Perigee(rad):   -2.961498260e+00

Mean Anom(rad):             -2.257209301e+00

Af0(s):                      4.863739014e-05

Af1(s/s):                    3.637978807e-12

week:                        902

**** Week 902 almanac for PRN-22 ***********

ID:                          022

Health:                      000

Eccentricity:                9.774208069e-03

Time of Applicability(s):    4.055040000e+05

Orbital Inclination(rad):    9.424598217e-01

Rate of Right Ascen(r/s):   -8.217485359e-09

SQRT(A) (m^1/2):             5.153635254e+03

Right Ascen at TOA(rad):     1.156740665e+00

Argument of Perigee(rad):    1.895342618e-01

Mean Anom(rad):             -2.911064863e+00

Af0(s):                      3.862380981e-04

Af1(s/s):                    3.637978807e-12

week:                        902

**** Week 902 almanac for PRN-23 ***********

ID:                          023

Health:                      000

Eccentricity:                1.178359985e-02

Time of Applicability(s):    4.055040000e+05

Orbital Inclination(rad):    9.649662375e-01

Rate of Right Ascen(r/s):   -7.851755690e-09

SQRT(A) (m^1/2):             5.153566406e+03

Right Ascen at TOA(rad):    -1.916701913e+00

Argument of Perigee(rad):   -2.122141361e+00

Mean Anom(rad):             -2.586663485e+00

Af0(s):                      1.239776611e-05

Af1(s/s):                    0.000000000e+00

week:                        902

**** Week 902 almanac for PRN-24 ***********

ID:                          024

Health:                      000

Eccentricity:                7.456302643e-03

Time of Applicability(s):    4.055040000e+05

Orbital Inclination(rad):    9.845005274e-01

Rate of Right Ascen(r/s):   -7.508884181e-09

SQRT(A) (m^1/2):             5.153604980e+03

Right Ascen at TOA(rad):    -2.989380598e+00

Argument of Perigee(rad):   -1.889477134e+00

Mean Anom(rad):              3.537368774e-02

Af0(s):                      3.900527954e-04

Af1(s/s):                    4.729372449e-11

week:                        902

**** Week 902 almanac for PRN-25 ***********

ID:                          025

Health:                      000

Eccentricity:                6.454467773e-03

Time of Applicability(s):    4.055040000e+05

Orbital Inclination(rad):    9.379537702e-01

Rate of Right Ascen(r/s):   -7.886042930e-09

SQRT(A) (m^1/2):             5.153735352e+03

Right Ascen at TOA(rad):     9.995929897e-02

Argument of Perigee(rad):   -2.745729685e+00

Mean Anom(rad):              1.381591678e+00

Af0(s):                     -4.768371582e-06

Af1(s/s):                    0.000000000e+00

week:                        902

**** Week 902 almanac for PRN-26 ***********

ID:                          026

Health:                      000

Eccentricity:                9.697437286e-03

Time of Applicability(s):    4.055040000e+05

Orbital Inclination(rad):    9.574041963e-01

Rate of Right Ascen(r/s):   -8.068907320e-09

SQRT(A) (m^1/2):             5.153641113e+03

Right Ascen at TOA(rad):    -9.061591029e-01

Argument of Perigee(rad):   -4.351037741e-01

Mean Anom(rad):              2.281053782e+00

Af0(s):                     -1.602172852e-04

Af1(s/s):                   -7.275957614e-12

week:                        902

**** Week 902 almanac for PRN-27 ***********

ID:                          027

Health:                      000

Eccentricity:                1.219415665e-02

Time of Applicability(s):    4.055040000e+05

Orbital Inclination(rad):    9.421003461e-01

Rate of Right Ascen(r/s):   -7.840326610e-09

SQRT(A) (m^1/2):             5.153715820e+03

Right Ascen at TOA(rad):     1.163675711e-01

Argument of Perigee(rad):    2.987223387e+00

Mean Anom(rad):             -6.872196198e-01

Af0(s):                      2.670288086e-05

Af1(s/s):                    0.000000000e+00

week:                        902

**** Week 902 almanac for PRN-29 ***********

ID:                          029

Health:                      000

Eccentricity:                5.472183228e-03

Time of Applicability(s):    4.055040000e+05

Orbital Inclination(rad):    9.537789226e-01

Rate of Right Ascen(r/s):   -8.126052720e-09

SQRT(A) (m^1/2):             5.153371582e+03

Right Ascen at TOA(rad):    -9.310008883e-01

Argument of Perigee(rad):   -1.923468471e+00

Mean Anom(rad):              6.864174604e-01

Af0(s):                      9.536743164e-06

Af1(s/s):                    0.000000000e+00

week:                        902

**** Week 902 almanac for PRN-30 ***********

ID:                          030

Health:                      000

Eccentricity:                5.289077759e-03

Time of Applicability(s):    4.055040000e+05

Orbital Inclination(rad):    9.526703954e-01

Rate of Right Ascen(r/s):   -8.114623640e-09

SQRT(A) (m^1/2):             5.153731445e+03

Right Ascen at TOA(rad):     1.181831837e+00

Argument of Perigee(rad):    1.635033607e+00

Mean Anom(rad):             -2.796348810e+00

Af0(s):                     -5.054473877e-05

Af1(s/s):                   -3.637978807e-12

week:                        902

**** Week 902 almanac for PRN-31 ***********

ID:                          031

Health:                      000

Eccentricity:                7.433891296e-03

Time of Applicability(s):    4.055040000e+05

Orbital Inclination(rad):    9.617244601e-01

Rate of Right Ascen(r/s):   -8.274630758e-09

SQRT(A) (m^1/2):             5.153591309e+03

Right Ascen at TOA(rad):     2.222844601e+00

Argument of Perigee(rad):    7.096163034e-01

Mean Anom(rad):              1.044998169e+00

Af0(s):                      3.242492676e-04

Af1(s/s):                    1.818989404e-11

week:                        902

--------------------Cut HERE----------------------------

This program has been tested on several PC's running MS-DOS and/or

Windows 95.  It has also been tested on a Hewlett Packard 200LX palmtop.

The only Garmin unit available for testing this program is my own GPS45,

software revision 2.43, therefore the only upload/download functions in

this program are those known to be supported by the GPS45, Software

Version 2.43.

Versions of G7TO greater than 2.04 have been tested on my Garmin 12XL,

version 2.01, only.  I no longer own a GPS45.

Ron Henderson WA7TAS

crh@cv.hp.com